A Confidence-Driven Probabilistic Machine Learning Framework for Electricity Demand–Supply Gap Forecasting and RAG-Based Energy Planning

Dr.Kamatchi K.S 
Associate Professor

Department of InformationTechnology

Sathyabama Institute of Science and Technology.Chennai kamatchi.k.s.it@sathyabama.ac.in

Udayakumar S

UG scholar 
Department of Information Technology Sathyabama Institute of Science and Technology.Chennai
udhayakumar0101@gmail.com

Tamilarasu D 
UG scholar 
Department of Information Technology Sathyabama Institute of Science and Technology.Chennai tamilhackerd@gmail.com

Abstract— This study addresses the research gap in electricity demand and supply forecasting. Most of the existing papers and  studies cover only the electricity demand forecasting or energy generation forecasting. This study proposes a Confidence-Driven Probabilistic Machine Learning Framework for Electricity Demand–Supply Gap Analysis and RAG-Based Energy Planning for Tamil Nadu, India.  In our project The framework analyzes past electricity demand, power generation from different energy sources, and weather conditions to forecast the electricity demand and supply for the next three months and we getting the historical data’s and source-wise electricity. The sources are (coal, thermal, hydro, solar,  and other sources), and we using weather parameters such as temperature, humidity, rainfall and solar irradiance. We going to develop separate  machine learning models for electricity demand and electricity supply. The Machine learning models are Random Forest, XGBoost, LightGBM, LSTM. Different forecasting techniques are implemented and assessed using standard evaluation metrics that are use in this model such as MAE, RMSE, MAPE and R². The algorithm that demonstrates the best predictive performance is chosen as the final forecasting model. To make the forecasts more accurate and reliable, confidence intervals are used to measure the uncertainty in the predictions. The calculated prediction intervals at 90%, 95%, and 99% confidence levels  forecasts. The forecasted electricity demand and supply are compared to identify possible shortages or excess power. A Retrieval-Augmented Generation (RAG) module is also used to provide useful energy planning recommendations based on energy policies and existing knowledge. This framework helps improve forecasting, supports better planning, and promotes efficient use of electricity.

Keywords— Machine Learning, Demand–Supply Gap Analysis, Electricity Forecasting, Confidence Intervals, RAG, Energy Planning.

# Introduction

Electricity is one of the most important things in our day to day life for industrial developments, our

houses and modern life. Nowadays the demand of electricity is increasing due to the increasing number of electric vehicles, increasing digital infrastructure etc. in the world. It will help to lower the green house gas emissions whenever the electricity demand also rises in the same time when renewable energy sources like solar or wind are being fast integrated into the power systems to access carbon neutrality. The renewable energy generation's environmental benefits are very weather-dependent. The changes have made the operation of the power system more difficult, balancing of the grid, long-term energy planning. We need to make the forecasting accurate the electricity demand and supply will ensure it have stable  grid operation. Over the years, the electricity demand forecasting has been enhanced from the old traditional statistical and autoregressive approach to the newer machine learning model and deep learning model approaches. Energy suppy integration and changing electricity[1] are added to the changed relationships. Further, some meteorological elements like temperature, humidity and seasonality are identified as significant factors of electricity demand. Some studies have demonstrated that assimilation of the weather information can improve the weather forecast in extreme weather and when electricity demand is high[1], [6]. Forecasting renewable supply is also critical as the volume of solar and wind generation relies on weather conditions and only occurs when resources are available. Forecasts can support operators to lower unutilised energy, facilitate energy storage planning and store grid stability [2]. Most machine learning and deep learning models are deterministic, which means that they do not provide a measure of uncertainty and have improved the accuracy of forecasting. This is addressed by probabilistic forecasting, which generates prediction intervals [3] to capture uncertainty, and recent Transformer-based probabilistic models and Monte Carlo Dropout techniques [3]–[5] which add confidence scores for reflecting real-world uncertainty. Despite this, prior research is disjointed with some focusing on demand forecasting or others on supply forecasting, and probabilistic forecasting has been done independently from combined demand–supply modeling [1]–[2], [3]–[5]. There are long-term demand–supply studies, but these are mostly based on scenario analysis, and do not provide uncertainty estimation through machine learning. RAG has also been employed to enhance the factual consistency of power-system question answering using LLMs [8]–[10], but with the exclusion of the actual forecast output in planning. To date, there is no common system for forecasting demand and supply, measuring uncertainty and facilitating intelligent energy planning. In this regard, this work attempts to contribute to this field with a Confidence-Driven Probabilistic Machine Learning Framework for Electricity Demand–Supply Gap Forecasting and RAG-Based Energy Planning. The framework relies on historical electricity and weather data to predict demands and supplies, calculate the demand and supply mismatch and provide measures of uncertainty in the form of confidence intervals with varying levels. All of these forecasts are then used together with trusted energy-domain knowledge in a RAG module to produce reliable, explainable planning recommendations, providing a more holistic approach to power-system planning and operation.

II Literature Survey

A. Energy Demand Forecasting

Demand forecasting plays a crucial role in generation scheduling and balancing supply and demand. With the increasing complexity of the grid, early techniques such as ARIMA, regression and support vector machines were too simple to describe the nonlinearity and volatility of the actual demand data, reducing their accuracy [7] [8]. In this, deep learning was used to decompose the demand signals into trend and noise parts, similar to the ECEEMDAN-BiGRU model [9]. The long-horizon accuracy was further improved in transformer-based methods, including TPEDF [10] and PMS-EDF [11] which also estimated uncertainty. Furthermore, integrating weather data such as weather and rainfall into LSTM and Transformer models has also enhanced their accuracy under sudden demand changes [12] and [13]. However, research on demand forecasting continues to be independent of supply forecasting and there is no way to estimate the difference between these and/or offer confidence intervals for planning purposes.

B. Energy Supply Forecasting

Unlike demand forecasts, renewable supply forecasts are more difficult because the supply from renewables is directly affected by weather conditions, making the scheduling and stability of the grid even more difficult [14], [15]. Weather-derived LSTM models are now better at forecasting solar and wind generation [14] and AutoML frameworks have been devised to automatically optimize and select features [15]. Spatial models like graph neural networks and LSTM [16] and compressed weather-based spatial features [17] have led to advances in regional and probabilistic forecasts. Multi-source deep learning models are more effective in extreme weather [18, 19]. But this research still takes a regional approach of forecasting solar or wind energy without considering the bond with energy demand forecasting or the mismatch between energy demand and energy supply.

C. Probabilistic Forecasting and Confidence Interval Estimation.

In addition, the use of probabilistic forecasting, that is, the generation of a range of forecast outcomes with confidence intervals, has received attention [20],[21] as a way of providing greater insight into forecasting reliability than a single point forecast. This is used in demand forecasting in TPEDF [20] and PMS-EDF [21] and similar techniques are used to enhance the scheduling and reserve allocation of the supply side [22, 23]. A prominent research work uses a joint probability distribution to forecast demand and supply side together and improves over deterministic approaches [24]. Uncertainty can be quantified with broader methods such as quantile regression [25] and Bayesian methods [26] or Monte Carlo sampling. Despite that, most work handles address the estimation of the demand–supply gap individually, the estimation of this gap together with confidence intervals is little explored, and none of this is related to decision-support systems.

D. Demand–Supply Gap Forecasting and RAG Based Energy Planning

Although grid stability relies on a balance between the demand and supply, most studies still approach them independently [27] and [28]. A few exceptions are: (1) A combined model for both demand and generation is presented in [24] with common distributions of probability; and (2) Probabilistic supply forecasting in combined heat and power dispatch is presented in [22]. Both, however, do not directly predict the difference between power needed and available. In isolation, studies have focused on power-system decision support applications involving large language models with retrieval-augmented generation (RAG), where the outputs are based on retrieved information instead of unrestrained generation. The same is true with energy dispatch [29] and related systems can seek to minimize factual errors and enhance contextual accuracy [30]. The improvement of the RAG methods includes multi-pass retrieval [31], multi-model cross-verification [32] and fine-tuning for dispatch regulations [33]. While there has been some work done in both of these areas, the two are not connected, with no current work that connects probabilistic demand–supply gap forecasts to confidence intervals to a RAG system that can suggest operator actions. Inspired by this gap, the authors present the present study that combines probabilistic demand forecasting, supply forecasting and RAG-based planning with gap estimation with confidence intervals (CI) in a single framework [24], [29]–[33].

III Methodology

3.1 Data Collection

The datasets were collected from government and public sources, including NITI Aayog, NWDP, and NASA POWER. They contain monthly electricity demand, generation, and weather data from April 2015 to December 2025, covering 129 records. The data was reviewed, aligned, and integrated for demand and supply forecasting, gap analysis, and RAG-based energy planning.

| Dataset | Source |
| --- | --- |
| Electricity Demand | NITI Aayog |
| Electricity Supply | NITI Aayog |
| Temperature | NASA Power |
| Humidity and Rainfall | NASA Power |
| Solar Irradiance | National Water Data Platform(NWDP) |

3.2 Data Refinement and Feature Development

After data collection, the datasets were preprocessed and integrated for machine learning. Inconsistent date formats were corrected, and hourly solar irradiance data was aggregated into daily and monthly averages to match the monthly frequency of the other datasets. The datasets were then merged using the Date column. Temporal and seasonal features, including Year, Month, Month_sin, and Month_cos, were created, while a Festival feature was included to represent festival-related variations in electricity demand. The final demand feature set consisted of weather, temporal, seasonal, and festival variables. The data was then divided chronologically into training and testing sets to preserve the time sequence.

3.3 Electricity Demand Forecasting

Four machine learning models,Random Forest, XGBoost, LightGBM, and LSTM,were developed for electricity demand forecasting. The models used weather variables, calendar features, festival information, and historical demand patterns as input features. Historical demand was represented using 1-, 2-, and 3-month lag values and 3-, 6-, and 12-month rolling averages, while Month was represented using sine and cosine transformations to capture seasonal patterns. The models were trained using a chronological split, with the last 24 months reserved for testing. Performance was evaluated using MAE, RMSE, MAPE, and R². The models achieved R² scores of 0.31, 0.34, 0.29, and 0.78 for Random Forest, XGBoost, LightGBM, and LSTM, respectively. LSTM achieved the best performance and was selected as the final demand forecasting model.

3.4 Electricity Supply Forecasting

The same four machine learning models—Random Forest, XGBoost, LightGBM, and LSTM—were developed to forecast future electricity supply. The models used lagged generation features from coal, oil and gas, nuclear, hydro, solar, wind, small hydro, and bio-power, along with Year, Month, and Quarter features. Historical total supply was represented using 1-, 3-, 6-, and 12-month lag values and 3-, 6-, and 12-month rolling averages to capture short- and long-term generation patterns. The data was divided chronologically, with the final 24 months reserved for testing. The models achieved R² scores of 0.31, 0.31, 0.38, and 0.56 for Random Forest, XGBoost, LightGBM, and LSTM, respectively. LSTM achieved the best overall performance and was selected as the final supply forecasting model.

3.5 Prediction Interval Estimation

A After selecting the final forecasting models, prediction intervals were generated to quantify uncertainty around the point forecasts. Historical forecasting residuals up to December 2025 were used to estimate the residual standard error σe, avoiding the use of future 2026 observations. The prediction interval at confidence level 1-αwas calculated as

PI1-α​=y​-z1-α/2​σe​,y​+z1-α/2​σe​

where yis the point forecast and z1-α/2is the standard normal critical value. Using z=1.645, 1.960, and 2.576, 90%, 95%, and 99% prediction intervals were generated for both electricity demand and supply.

3.6 Demand–Supply Gap Analysis

The demand–supply gap was calculated as the difference between predicted electricity demand and predicted supply:

Gt=Dt-St

where Gtis the demand–supply gap, Dtis the predicted demand, and Stis the predicted supply. A positive gap indicates a potential shortage, while a negative gap indicates excess supply. The 95% prediction interval was also considered to represent the uncertainty associated with the estimated gap.

3.7 RAG-Based Energy Planning Framework

After demand–supply gap analysis, a Retrieval-Augmented Generation (RAG) module was integrated to convert forecasting results into energy planning recommendations. The predicted gap was classified into three risk levels: below 3000 MU (Low), 3000–4500 MU (Moderate), and above 4500 MU (High). The RAG knowledge base comprised nine official energy-sector documents, including three Tamil Nadu and six national-level sources, containing 1,725 pages processed into 1,155 chunks. The forecasting output was converted into dynamic queries and transformed into 384-dimensional L2-normalized embeddings using all-MiniLM-L6-v2. The embedded documents were indexed using FAISS IndexFlatIP, with cosine similarity used to retrieve the top-five relevant chunks. The retrieved context, together with the predicted demand, supply, gap, and risk level, was provided to the Gemini LLM to generate grounded recommendations with source and page references. Thus, the RAG module connects ML-based forecasts with evidence-based policy recommendations for practical energy planning.

IV. Results and Discussion

4.1 Model Development and Evaluation

The proposed framework was implemented in Python 3.12 using Jupyter Notebook. Pandas and NumPy were used for data processing, while Random Forest, XGBoost, LightGBM, and LSTM models were implemented using Scikit-learn, XGBoost, LightGBM, and TensorFlow/Keras. The dataset was divided into 80% training and 20% testing sets using a time-based split to preserve chronological order. Model performance was evaluated using MAE, RMSE, MAPE, and R². The best-performing models were then used to generate prediction intervals, demand–supply gap estimates, and RAG-based energy planning recommendations.

4.2 Electricity Demand Forecasting Results

| Model | MAE | RMSE | MAPE (%) | R² Score |
| --- | --- | --- | --- | --- |
| Random Forest | 741.63 | 909.69 | 6.56 | 0.31 |
| XGBoost | 619.18 | 724.51 | 5.56 | 0.34 |
| LightGBM | 634.18 | 748.24 | 5.66 | 0.29 |
| LSTM | 368.72 | 412.93 | 3.41 | 0.78 |

Figure 1: Electricity Demand Forecasting

The used four machine learning models performance regarding electricity demand forecasting is shown in Table 2. The Mean Absolute Error (MAE), Root Mean Square Error (RMSE), Mean Absolute Percentage Error (MAPE) and R2 score were used to test the models. Among all the models, LSTM performed best with the maximum value of R² and minimum error values, making it the best model for predicting the demands of electricity in the future. Thus, LSTM is chosen for future steps of the proposed framework.

4.3 Electricity Supply Forecasting Results

| Model | MAE | RMSE | MAPE (%) | R² Score |
| --- | --- | --- | --- | --- |
| Random Forest | 823.30 | 1220.9 | 6.89 | 0.31 |
| XGBoost | 847.76 | 1215.3 | 7.13 | 0.31 |
| LightGBM | 802.74 | 1157.4 | 6.74 | 0.38 |
| LSTM | 731.56 | 973.93 | 6.30 | 0.56 |

Figure 2: Electricity Supply Forecasting Results

A comparison of the performance of the four machine learning models for electricity supply forecasting is shown in Table 3. The performance  metrics of the models were the same: MAE, RMSE, MAPE and R2 score. The results reveal LSTM was able to secure the highest R² score and the lowest prediction error, thereby performing the best. Based on this, LSTM was chosen as the final model to forecast electricity supply.

4.4 Prediction Interval Results

| Month | Predicted Gap(MU) | 95% PI | Status |
| --- | --- | --- | --- |
| Jan 2026 | 2237.88 | [138.99,4336.77] | Shortage |
| Feb 2026 | 3078.30 | [979.41,5177.19] | Shortage |
| Mar 2026 | 3020.25 | [921.36,5119.14] | Shortage |

The demand–supply gap was calculated from the predicted electricity demand and supply. The forecasted gaps for January, February, and March 2026 were 2237.88 MU, 3078.30 MU, and 3020.25 MU, respectively, indicating potential shortages. The 95% prediction intervals represent the uncertainty associated with the forecasted gaps.

4.4 RAG Evaluation Metrics Results

| RAG System | Context Precision (%) | Context Recall (%) | MRR. | nDCG@5 |
| --- | --- | --- | --- | --- |
| Proposed RAG | 56.00 | 66.25 | 0.66 | 0.7216 |

The proposed RAG system achieved 56.00% Context Precision and 66.25% Context Recall, indicating effective retrieval of relevant information. The MRR of 0.66 and nDCG@5 of 0.7216 demonstrate that relevant information was generally ranked within the top retrieved contexts. Overall, the results demonstrate effective retrieval and ranking of domain-specific energy information.

V. Conclusion

This study presents a unified framework for electricity demand and supply forecasting, uncertainty analysis, gap assessment, and RAG-based energy planning recommendations for Tamil Nadu. Among the evaluated models, LSTM achieved the best forecasting performance. The integrated RAG framework further supports evidence-based energy planning. Future work will explore real-time data, additional weather features, and Transformer-based models.

VI. References

[1] Shivam Sharma, Anamika Goel, Vedang Tiwari and Shiv Pratap Singh Waghel, "Electricity Demand Forecasting: A Review of Statistical, Machine Learning, Deep Learning and Hybrid Approaches,"DOI:10.1109/ACET67282.2025.11430367.

[2] Stephen J. Lee and Cailinn Drouin,"Forecasting Residential Heating and Electricity Demand with Scalable, High-Resolution, Open-Source	Models," DOI: 10.1016/j.egyai.2026.100726.

[3] Anber Abraheem Shlash Mohammad, Badrea Al Oraini, Suleiman Ibrahim Mohammad, Sultan Alaswad Alenazi, Torki M. Al-Fawwaz and Asokan Vasudevan, "Mathematical and Statistical Modelling of Electricity Demand Forecasting Using Artificial Neural Networks and SARIMA: Implications for Energy Supply Chain Planning," DOI: 10.1016/j.aej.2026.01.046.

[4] Bo Jie, Jumpei Baba and Akiko Kumada, "Mathematical Analysis of Time-series & Meteorological Factors in Electricity Demand Forecasting Based on Carbon Neutrality," DOI: 10.1109/ICPSASIA58343.2023.10294862.

[5] Sylvester William Chisale, Han Soo Lee and Manuel Alejandro Soto Calvo, "Strategic Forecasting of Electricity Demand for 100% Electrification in Malawi by 2063: A Data-Driven ECEEMDAN-BiGRU and Quantile Regression Approach," DOI: 10.1016/j.energy.2025.137212.

[6] Difei Miyao and Masaki Nakamura, "Time and Day-Based Peak Electricity Demand Forecasting: A Comparative Analysis of Machine Learning Models for Peak Cut and Decarbonization," DOI: 10.1109/ICRERA62673.2024.10815184.

[7] Ligang Hou, Xin Shen and Liqi Zhang, "Urban Electricity Demand Forecasting with a Hybrid Machine Learning Model,"DOI: 10.1109/ICNSC58704.2023.10319027.

[8] Zhanxing Xu, Lingjun Xu, Zhanzhi Liu and Qiang Liu, "Analysis of Pumped Storage Hydropower Demand-Supply Gap in East China," DOI: 10.1109/ICCEPE66357.2025.11193283.

[9] Jiaxi Li, Ming Wen, Bo Wen, Zongchao Yu, Yue Qin and Xinyang Zhang, "Multi-objective Optimization of Power Supply and Demand Balance Considering the Impact of Coal Prices on Coal-Fired Power Generation Willingness," DOI: 10.1109/EI264398.2024.10990690.

[10] Wang Lening, Sun Dagen, Li Dong, Yang Chaoran, Duan Hengcheng, Li Jiao and He Zhonghai, "Electricity Demand Forecasting via Multi-Scale Structure-Guided Transformer with Adaptive Paths," DOI: 10.1109/ICCWAMTIP64812.2024.10873637.

[11] Feng Xu, Yi Lu, Peng Qiu and Chao Ding,"Flexible Combined Heat and Power Dispatch Based on Probabilistic Prediction of Wind Power," DOI: 10.1109/AEEES56888.2023.10114217.

[12] Behrouz Banitalebi, Srimantoorao S. Appadoo, Yuvraj Gajpal and Aerambamoorthy Thavaneswaran, "Intelligent Probabilistic Forecasts of Day-Ahead Electricity Prices in a Highly Volatile Power Market," DOI: 10.1109/COMPSAC51774.2021.00034.

[13] Guillermo Terrén-Serrano, Ranjit Deshmukh and Manel Martínez-Ramón,"Probabilistic Day-Ahead Forecasting of System-Level Renewable Energy and Electricity Demand," DOI: 10.1038/s41467-026-69015-w.

[14] Haisheng Hong, Zhixuan Zhao, Zhifang Zhu, Zheng Sun, Shengkai Chen and Zihao Weng, "Probabilistic Analysis Enabled Proactive Fault Risk Assessment of Electricity Power Distribution Systems," DOI: 10.1109/PEEEC63877.2024.00013.

[15] Yang Cao, Zhenzhen Dang, Feng Wu, Xovee Xu and Fan Zhou, "Probabilistic Electricity Demand Forecasting with Transformer-Guided State Space Model,"	DOI: 10.1109/AUTEEE56487.2022.9994294.

[16] Mingjian Ni, Jing Zhang, Cong Fu, Jiaqi Wang, Xin Ning and Siyuan Li, "ChatGrid: Intelligent Knowledge Q&A for Power Dispatching Control Based on Large Language Models and Retrieval-augmented Generation," DOI: 10.1109/ITNEC60942.2024.10733337.

[17] Rui Wang, Qingyu Wang, Yalin Nan, Hang Ye and Panbo He, "Construction and Evaluation of a Large Language Model for the Energy Domain Leveraging Retrieval-Augmented Generation," DOI: 10.1109/ICFTIC68075.2025.11324983.

[18] M. Harish, S. Suganth, S. Sritharan and R. Nithiavathy, "Generative AI Powered Chatbot for Energy Consumption Optimization Using Retrieval-Augmented Generation," DOI: 10.1109/ICISCN67954.2026.11566088.

[19] Shuai Zhao, Jun Feng, Xiaojun Shen, Huaguang Yan, Zhenyu Chen, Jianguang Du, Pengxin Luo and Zhen Wang, "Knowledge Retrieval Enhancement for Power System Based on Adaptive Dynamic Text Segmentation and Hierarchical Multi-Query," DOI: 10.1109/PSGEC66102.2025.11151081.

[20] Xin Huang, Yibo Chen, Xiaowen Feng, Hongyu Zhu and Chao Tang, "Multi-Agent Cooperative Retrieval-Augmented Generation for Efficient Electric Power Knowledge Retrieval Quiz," DOI: 10.1109/ICPEA67476.2025.11349299.

[21] Ying Zou, Ying Liu, Sai Zhang, Bochuan Song and Xiao Liang, "Robust Fine-Tuning of Large Language Models for Retrieval Augmented Generation in Power Grid Dispatching Regulations," DOI: 10.1109/POWERCON66300.2025.11295460.

[22] Jichao Ye, Linping Liu, Yonghai Xu, Tong Xia, Shiwang Yang and Xinwei Hu, "Distributed Photovoltaic Power Forecasting Based on Auto Machine Learning Framework and Fusion Perception Technology," DOI: 10.1109/ICGEPS65133.2025.11034521.

[23] Rui Zhang, Yanning Wang and Yujie Tang, "Enhancing Agent-Based Electricity Purchase Forecasting Accuracy Comprehensively Supports Power Supply and Electricity Spot Trading," DOI: 10.1109/ICEPET61938.2024.10625854.

[24] Cheng Jifeng, Sun Junjie, Wang Li, Wu Zhenghao, Wang Yuqing and Wang Fei, "Optimization Strategy for Ultra-short-term Photovoltaic Power Forecasting Based on the Fusion of Day-ahead Forecast and Intraday Real-time Information," DOI: 10.1109/IAS62731.2025.11061420.

[25] Shumin Sun, Shibo Wang, Yan Cheng, Liqun Sun, Xiaoqi Zhang and Yunhai Lv, "Photovoltaic Station Power Forecasting for Integrated Energy Systems Based on Spatiotemporal Features," DOI: 10.1109/CEEPE64987.2025.11034336.

[26] Hyeonwoo An, Guwon Yoon, Keonhee Cho, Yahui Dong, Seunghwan Kim and Sehyun Park, "Renewable Energy Generation Forecasting Model for Reducing Power System Load and Solving Output Control Issues," DOI: 10.1109/APACE62360.2024.10877341.

[27] Xiaoli Zhu, Xuannian Lei, Haiyi Xiao, Hang Xu and Zhiyuan Zhu, "Research on Multi-Scale Forecasting Model of Wind Power in Extreme Weather Based on Multi-Source Heterogeneous Data Fusion," DOI: 10.1109/AECSPE66597.2025.00036.

[28] Jia Sun, Zehui Wang, Xiaozhuo Guan, Pingjian Yang, Xuesong Qi, Zhenxin Li, Li Tian and Zhen Wu, "Research on Ultra-Short-Term Wind Power Generation Forecasting Based on Expert System Method and RBF Neural Network Prediction Model," DOI: 10.1109/ICESEP62218.2024.10651817.

[29] Sajib Debnath, Uzzal Basu, Md Abubakkar, Md Robiul Islam, Mithu Debnath and Ashis Kumer Biswas, "Extreme Weather Grid Load Forecasting using Weather-Informed LSTM and Transformer Machine Learning Models," DOI: 10.1109/NAPS66256.2025.11272315.

[30] Georgios Mitrentsis, "From High-Dimensional Weather Data to Accurate Renewable Generation Forecasts: A Statistical Aggregation Method," DOI: 10.1109/EEM68581.2026.11589794.

[31] Shani Abeywickrama and P. P. G. Dinesh Asanka,"Integrating Weather Patterns into Machine Learning Models for Improved Electricity Demand Forecasting in Sri Lanka," DOI: 10.1109/SCSE59836.2023.10215047.

[32] Indra A. Aditya and Didit Adytia, "Modelling of Spatially Correlated Weather-Based Electricity Forecasting Using Combined Frequency-Based Signal Decomposition with Optimized Boosting Approach," DOI: 10.1016/j.ijepes.2025.110698.

[33] Rishi Malyala, Abhijit Palakurthy, Anam Malik and Jayashri Ravishankar, "Confidence Interval Calibration for Transformer-GRU Forecasting Models using Monte Carlo Dropout," DOI: 10.1109/ETFG61999.2025.11402342.